Selecting suitable plant species for airborne particulate matter (PM) pollution management in an urban area

Ankesh Tiwari1 & Sudhir Kumar Pandey1*

1.Department of Botany, Guru Ghasidas Vishwavidyalaya, (A Central University) Koni, Bilaspur (C.G.), 495009, India

*Email – skpbhu@gmail.com; ankeshtiwari@icloud.com

Abstract

One of the most serious issues for the health of both people and the environment in developing nations is pollution. Particulate matter (PM) concentrations are rising in metropolitan areas, mostly as a result of industrialization, unplanned urbanization, an alarming increase in the fleet of cars, population growth, and poor long-term planning. In this category, PM with a size less than 10 µm (PM10) and 2.5 µm (PM2.5) should be focused more on considering their respirable size range and the impact on human health. In this study, six plant species (Polyalthia longifolia, Dalbergia sissoo, Syzygium cumini, Alstonia scholaris, Senna siamea, and Pongamia pinnata) were studied at two industrial sites of Bilaspur city (NTPC (National Thermal Power Corporation), Seepat and the other at Sirgitti industrial area), for their PM retention capacity in different size fractions. This involved the processing of images obtained using scanning electron microscopy (SEM) and a new method of PM measurement. We found that the amount of PM was highest in the case of Syzygium cumini at site 1 and Alstonia scholaris at site 2 by applying the traditional gravimetric approach. However, when the PM counting was done using the new approach based on SEM images, Dalbargia sissoo and Senna siamea were found to be the best plant species at both sites for PM retention. These plants have the capacity to hold PM in the respirable range (10 µm) because of their unique micromorphological characteristics. EDS (energy dispersive spectroscopy) was also performed to identify the source of PM bound metals. Hence, the present study reveals that before recommending plants for an urban greening program, one should consider the PM retention abilities of plants using the method adopted in this study.

Key words: Respirable particulate matter (RSPM); industrial areas; Ecomanagement; Scanning electron Microscopy (SEM); Criteria air pollutant.