Phycoremediation potential of cyanobacterial species Oscillatoria sp. And Phormidium sp. for treating wastewater collected from drainage.

Shristi Chahal and Rupali Bhandari*

Botany Discipline

School of Biological Sciences and Biotechnology

Goa University, Goa, India, 403206

Email: rupali.bhandari@unigoa.ac.in

Environmental pollution is one of the most pressing global issues and requires attention. Urbanization, Industrialization, and natural processes have accelerated the level and pace of toxic pollutants, such as heavy metals, dyes, pesticides, phenols, organic compounds, etc., in the aquatic system leading to water pollution. The classical treatments of wastewater include physical, chemical, and biological methods. The known chemical and physical methods require much energy and induce secondary pollutants into the environment. Phycoremediation is a biological process of removing waste from wastewater using algae. Microalgae have played a crucial role in removing nutrients such as nitrogen, phosphorous, sulphur and minerals that act as feed for the growth of these algae rather than being contaminants. The preliminary study on the treatment of wastewater collected from the drainage site of Goa using two algae, Oscillatoria sp. and Phormidium sp., showed a decrease in the physicochemical properties of wastewater after treatment. On treatment with Oscillatoria sp., a neutral pH range was observed with a reduction in nitrite content (82%), ammonia content (81%), phosphate content (79%), sulfate concentration (64%), total hardness (61%), and total dissolved solids (67%) after inoculation of the algae. However, on the treatment of wastewater with Phormidium sp., a reduction of nitrite content, ammonia content, phosphate content, sulfate concentration, total hardness, and total dissolved solids (TDS) was found to be 43%, 60%, 67%, 50%, 58%, and 49%  respectively with a neutral pH range. On a comparative basis, Oscillatoria sp. showed better results than Phormidium sp. Therefore, it can be concluded that both Oscillatoria sp. and Phormidium sp. have phycoremediation potential to treat the wastewater.